# CENTRAL PUBLIC WORKS DEPARTMENT (CPWD)

NIT Reference: CPWD/EE/ELECT/2026/048
Date of Issue: 15-September-2026

NOTICE INVITING TENDER FOR INTERNAL ELECTRIFICATION WORKS AT NEW SECRETARIAT COMPLEX

# Section I: Eligibility & General Conditions

1.1 The bidder must be an authorized class-1 electrical contractor registered with CPWD or State PWD.
1.2 Earnest Money Deposit: INR 2,50,000 to be deposited via electronic RTGS/NEFT.
1.3 Bids shall remain valid for 90 days from the date of technical bid opening.

# Section IV: Technical Specifications and Schedule of Requirements

4.1 Wiring Cables: Supply and laying of 1100V working voltage grade PVC insulated single-core and multi-core flexible copper conductor building wires with Flame Retardant Low Smoke (FRLS) insulation. Conductor purity must conform to electrolytic tough pitch copper.
4.2 Distribution Switchgear: Miniature Circuit Breakers (MCB) of breaking capacity 10 kA, C-curve characteristics, for 16A and 32A lighting and power circuits. Sub-main boards must include 30mA residual current protection for shock prevention.
4.3 Conduit System: Rigid plain non-metallic PVC conduits of heavy mechanical stress class for concealed wall and ceiling wiring.
4.4 Plugs & Sockets: 6A and 16A modular 3-pin shuttered socket-outlets with copper alloy contacts.

# Section V: Commercial Terms and Payment Conditions

5.1 Payment terms: 80% against delivery and physical verification, 20% on successful installation.
5.2 Liquidated damages shall be applicable at 0.5% per week of delay up to a maximum of 10%.